You previously discussed developing a construction estimation and calculating app that incorporated structural engineering formulas.

Here are the key threads and calculation logic you explored:

- Foundation & Concrete: Logic for foundation footings and rebar requirements.

- Retaining & Wall Loading: Bending moment calculations for basement walls under lateral soil pressure.

- Timber & Framing: Heavy timber truss design rules and roof framing calculations.

- CAD Integration: Building an add-on for AutoCAD (using C#) to read drawing parameters and automate structural estimations directly from blueprints.

Is there a specific formula or feature from that project you'd like to pick back up on?

For steel, we spent time working through calculations for two primary structural members: Wide Flange Beams (I-Beams) and Square/Rectangular Steel Tubing (HSS).

Here are the structural formulas and logic we covered for steel:

### 1. Bending Stress & Moment Capacity

To verify if a steel beam can handle a given load without exceeding allowable stress limits:

- Bending Stress Formula:

σ=MSx

  - σ: Calculated Bending Stress (psi or ksi)

  - M: Maximum Bending Moment (lb-in or k-in)

  - Sx: Elastic Section Modulus about the X-axis (in3), pulled from AISC steel tables for the specific shape.

- Allowable Biaxial Bending Check: Ensuring total combined stress stays within the allowable yield limit (σallowable≤0.66Fy for compact shapes, where Fy=50 ksifor standard A992 steel).

### 2. Maximum Deflection Calculation

For a simply supported steel beam with a uniform load (w in lb/ft):

- Deflection Formula:

Δmax=5wL4384EIx

  - E: Modulus of Elasticity for steel (29,000,000 psi)

  - Ix: Moment of Inertia (in4)

  - L: Span length (inches)

- Deflection Limits: Checking if calculated deflection meets code thresholds like L360(live load) or L240(total load).

### 3. Square Steel Tubing (HSS Columns)

For compression members (columns), we looked at:

- Slenderness Ratio: KLr(where r=I/Ais the radius of gyration).

- Euler Buckling Load: Calculating critical axial load capacity before lateral buckling occurs.

Are you looking to integrate these specific steel checks directly into your estimator or C# AutoCAD plugin next?